EXPLAIN AND BUILD A DEVICE THAT MONITORS AIR QUALITY (E.G., PM2.5, CO2 LEVELS) AND DISPLAYS THE DATA ON A CLOUD DASHBOARD

Here’s a full explanation and build guide for an Air Quality Monitoring Device that tracks PM2.5 and CO2 levels and displays the data on a cloud dashboard (using platforms like ThingSpeak or Blynk).

## ✅ Objective

Build a device to:

- Measure air quality parameters: PM2.5, CO2.

- Send data to the cloud using Wi-Fi.

- Display data on a dashboard that can be accessed remotely.

## 🧰 Components Required

| Component | Quantity |
| --- | --- |
| NodeMCU (ESP8266) or ESP32 | 1 |
| PMS5003 or SDS011 (PM2.5 sensor) | 1 |
| MH-Z19 or CCS811 (CO2 sensor) | 1 |
| Jumper Wires / Breadboard | As needed |
| USB Cable (for programming) | 1 |
| ThingSpeak or Blynk account | 1 |

## Circuit Connections

Example: Using ESP32 + PMS5003 + MH-Z19
(You can swap similar sensors)

PMS5003 (PM2.5):

- VCC → 5V

- GND → GND

- TX → GPIO16 (RX on ESP32)

- RX → GPIO17 (TX on ESP32)

MH-Z19 (CO2):

- VCC → 5V

- GND → GND

- TX → GPIO22

- RX → GPIO21

## 💻 Cloud Platform Options

### Option 1: ThingSpeak (Free and Easy)

- Go to thingspeak.com, create an account.

- Create a new channel.

- Enable fields for PM2.5, CO2.

- Note down the Channel ID and API Key.

## 🧠 Arduino Code Example (for ESP32 + ThingSpeak)

#include <WiFi.h>

#include "ThingSpeak.h"

#include <SoftwareSerial.h>

const char* ssid = "YourWiFiSSID";

const char* password = "YourWiFiPassword";

WiFiClient client;

// ThingSpeak

unsigned long channelID = YOUR_CHANNEL_ID;

const char* writeAPIKey = "YOUR_API_KEY";

// PM2.5 Sensor (PMS5003)

SoftwareSerial pmsSerial(16, 17); // RX, TX

// CO2 Sensor (MH-Z19)

SoftwareSerial co2Serial(21, 22); // RX, TX

void setup() {

Serial.begin(115200);

pmsSerial.begin(9600);

co2Serial.begin(9600);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000); Serial.print(".");

}

ThingSpeak.begin(client);

}

void loop() {

int pm25 = readPM25();

int co2 = readCO2();

ThingSpeak.setField(1, pm25);

ThingSpeak.setField(2, co2);

ThingSpeak.writeFields(channelID, writeAPIKey);

Serial.println("PM2.5: " + String(pm25) + " ug/m3");

Serial.println("CO2: " + String(co2) + " ppm");

delay(15000); // ThingSpeak requires 15s delay

}

int readPM25() {

if (pmsSerial.available() >= 32) {

uint8_t buf[32];

pmsSerial.readBytes(buf, 32);

int pm25 = buf[12] << 8 | buf[13];

return pm25;

}

return -1;

}

int readCO2() {

byte cmd[9] = {0xFF, 0x01, 0x86, 0, 0, 0, 0, 0, 0x79};

co2Serial.write(cmd, 9);

delay(100);

byte response[9];

co2Serial.readBytes(response, 9);

if (response[0] == 0xFF && response[1] == 0x86) {

int co2 = response[2] * 256 + response[3];

return co2;

}

return -1;

}

## 📊 Dashboard View

On ThingSpeak:

- You will see live graphs of PM2.5 levels and CO2 concentrations.

- You can set up alerts if levels exceed safe limits.

## 📸 Want the Circuit Diagram Image?

Let me know and I’ll generate the circuit image for this setup too.

## 🚀 Optional Enhancements

- Add temperature and humidity sensor (e.g., DHT22).

- Use OLED display for local data display.

- Store historical data to Google Sheets or Firebase.

- Add Buzzer or RGB LED for real-time air quality warnings.